/tmp/in/input.doc

# Roadside Vegetation Maintenance

Typical cross section *Not to scale

- Zone A

  - Spray – Vegetation Free

  - Spray width based on type of road

    - 2’ for local

    - 3’ for collectors

    - 5’ for arterials

- Zone B

  - Mow to ditch catch point (May – Oct.)

  - Brush every 7 years (Sept. – Oct.)

  - Spot-spray brush (e.g. poison oak, blackberries, Scotch broom)

  - Clean ditches (Approx. 15-yr. Cycle)

- Zone C – Behind Ditch

  - Maintenance free except the following:

    - Emergency and/or dangerous circumstances

    - Vision hazard as needed

    - Vegetation in zone “C” is encroaching into zone “A” or “B”, within 16 ft. vertical of maintenance (e.g. shoulder grading and ditch cleaning)

    - Tree limbs overhanging into the roadway or zone “A” will be trimmed flush to the beginning of zone “C”

*Except where vegetation management agreement exists with abutting property owners

*Rural vertical clearance 16”

*Urban vertical clearance 14”